- So the student id I have is 2608342 and according to the algorithm it gets converted to 260.83.42.0.

    - Since the first three digits of the newly formed ip address is more than 223 we reconfigure

    - the ip address as 193.83.42.0 which doesn’t happen to be the private ip address which will be the final block ip address we are going to use for this lab.

- As show in the table we need to configure 6 subnets out of which subnet A, B, C does not have any hosts meaning they are the point-to-point connection (connection between routers). So the project wants use to have separate router for each subnet. Altogether we are having 3 routers for this lab.

| Subnets | Number of Hosts |
| --- | --- |
| Subnet A | None |
| Subnet B | None |
| Subnet C | None |
| Subnet D | 1 server and 4 computers |
| Subnet E | 1 server and 18 computers |
| Subnet F | 1 server and 20 computers |

| Subnets | Number of Hosts | Comments | Nearest Decimal | Host Bits Needed | Prefix | Subnetting Order |
| --- | --- | --- | --- | --- | --- | --- |
| Subnet A | None | 2 routers | 4 | 2 | 30 | 6 |
| Subnet B | None | 2 routers | 4 | 2 | 30 | 5 |
| Subnet C | None | 2 routers | 4 | 2 | 30 | 4 |
| Subnet D | 1 server and 4 computers | 6 devices including a router | 8 | 3 | 29 | 3 |
| Subnet E | 1 server and 18 computers | 20 devices including a router | 32 | 5 | 27 | 2 |
| Subnet F | 1 server and 20 computers | 22 devices including a router | 32 | 5 | 27 | 1 |

- The above table has been expanded below to show the calculations leading up to making the decision of which order subnetting will be done, including the number of host bits reserved for the host ID for each subnet. The prefix will be calculated by subtracting the number of host bits needed from the total 32 bits contained in an IPv4 address.

- Table 3 below shows the network, first host, last host and broadcast IPv4 addresses for each subnet in the network. Using the subnetting order from Table 2 above, I will start with subnet F and the network address 195.39.92.0, I will then use the next IP (195.39.92.1) for the first host address. To calculate the last host address I will find the address 2n – 2 after the first host address. In this case, 2n – 2 represents how many addresses are allocated for network devices where n is the number of host bits needed as indicated from Table 2 above. This gives a last host address of 195.39.92.30 for subnet F. Finally, the broadcast address is calculated as the next address after the last host address (195.39.92.31). This process will be repeated for the subnets in the order designated from Table 2 above. Calculations for the Subnet Mask and Bit mask can be found below the table.

| Subnet | Network Address | Subnet Mask | First Host Address | Last Host Address | Broadcast | Bit Mask |
| --- | --- | --- | --- | --- | --- | --- |
| A | 193.83.42.80 | 255.255.255.252 | 193.83.42.81 | 193.83.42.82 | 193.83.42.83 | /30 |
| B | 193.83.42.76 | 255.255.255.252 | 193.83.42.77 | 193.83.42.78 | 193.83.42.79 | /30 |
| C | 193.83.42.72 | 255.255.255.252 | 193.83.42.73 | 193.83.42.74 | 193.83.42.75 | /30 |
| D | 193.83.42.64 | 255.255.255.248 | 193.83.42.65 | 193.83.42.70 | 193.83.42.71 | /29 |
| E | 193.83.42.32 | 255.255.255.224 | 193.83.42.33 | 193.83.42.62 | 193.83.42.63 | /27 |
| F | 193.83.42.0 | 255.255.255.224 | 193.83.42.1 | 193.83.42.30 | 192.83.42.31 | /27 |